# IFT 211 — Mid-Semester Lab Assessment

# Smart Water Tank Monitoring & Control System

| Course | IFT 211 — Digital Logic Design |
| --- | --- |
| Project | Smart Water Tank Monitoring & Control System |
| Submission type | Individual submission |
| Student name | ______________________________ |
| Matric number | ______________________________ |
| Simulator used | Logisim 2.7.1 (file also opens in Logisim Evolution) |
| Circuit file | circuit/water_tank.circ |

Design constraint honoured throughout: pure digital logic only — logic gates, D flip-flops, and wiring. No microcontrollers, no abstract black-box blocks.

# Table of Contents

- System Overview and Block Diagram

- Step 1 — Sensor Encoding (Combinational Logic)

- Step 2 — Comparator (Full Detection)

- Step 3 — Pump Control Logic

- Step 4 — Register (Level Memory)

- Step 5 — Overflow Counter and Alarm

- Step 6 — 7-Segment Display Decoder

- Boolean Derivations — Summary

- Testing Scenarios and Simulation Results

- System Explanation and Lab-Concept Mapping

- How to Open and Run the Circuit

# 1. System Overview and Block Diagram

A residential or industrial water tank must monitor its water level, indicate LOW / MEDIUM / FULL, stop filling automatically when full, display the level visually, and raise an alarm when there is an overflow risk. The system inputs are four water-level sensors plus CLOCK and RESET:

| Sensor | Meaning | Mounted at |
| --- | --- | --- |
| S0 | Empty-level probe | tank bottom |
| S1 | 25 % probe | quarter height |
| S2 | 50 % probe | half height |
| S3 | 100 % probe (Full) | tank top |

A sensor outputs logic 1 while water covers it, so a physically consistent reading is thermometer-coded: if a higher sensor is wet, every sensor below it is wet too. The five valid readings are:

| Water level | S3 S2 S1 S0 | Level name |
| --- | --- | --- |
| Dry tank | 0 0 0 0 | Empty (00) |
| Below 25 % | 0 0 0 1 | Empty (00) |
| 25 % | 0 0 1 1 | 25 % (01) |
| 50 % | 0 1 1 1 | 50 % (10) |
| 100 % | 1 1 1 1 | Full (11) |

All other 11 input combinations are physically impossible (non-thermometer) codes; Section 2 shows how they are used as don't-cares and Section 9 shows how the built circuit actually responds to them.

The implemented signal flow matches the block diagram required by the brief:

Sensors (S0–S3)

|

v

+----------------+

| Encoding Logic |            STEP 1

+----------------+

|

v

Level (L1 L0)

/      |       \

v       v        v

Pump Logic  Register  Comparator   STEPS 3, 4, 2

(NAND)    (2x D-FF)  (FULL=L1·L0)

|       |            |

v       |            v

PUMP pin   |     Overflow Counter  STEP 5

+ LED      |            |

|            v

|          ALARM (Q1·Q0) + LED

v

BCD → 7-Segment Decoder        STEP 6

|

v

7-Segment Display (0–3)

CLOCK drives the register and counter flip-flops; RESET asynchronously clears all four flip-flops.

# 2. Step 1 — Sensor Encoding

Goal: convert the four sensor bits into the 2-bit level code L1 L0 ∈ {00 Empty, 01 25 %, 10 50 %, 11 Full}.

## 2.1 Truth table (with don't-cares)

m is the minterm number of S3 S2 S1 S0. Valid thermometer rows are marked; every impossible row is a don't-care (×) that the K-maps may exploit.

| m | S3 | S2 | S1 | S0 | Valid? | L1 | L0 | Level |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | yes | 0 | 0 | Empty |
| 1 | 0 | 0 | 0 | 1 | yes | 0 | 0 | Empty |
| 2 | 0 | 0 | 1 | 0 | no | × | × | — |
| 3 | 0 | 0 | 1 | 1 | yes | 0 | 1 | 25 % |
| 4 | 0 | 1 | 0 | 0 | no | × | × | — |
| 5 | 0 | 1 | 0 | 1 | no | × | × | — |
| 6 | 0 | 1 | 1 | 0 | no | × | × | — |
| 7 | 0 | 1 | 1 | 1 | yes | 1 | 0 | 50 % |
| 8 | 1 | 0 | 0 | 0 | no | × | × | — |
| 9 | 1 | 0 | 0 | 1 | no | × | × | — |
| 10 | 1 | 0 | 1 | 0 | no | × | × | — |
| 11 | 1 | 0 | 1 | 1 | no | × | × | — |
| 12 | 1 | 1 | 0 | 0 | no | × | × | — |
| 13 | 1 | 1 | 0 | 1 | no | × | × | — |
| 14 | 1 | 1 | 1 | 0 | no | × | × | — |
| 15 | 1 | 1 | 1 | 1 | yes | 1 | 1 | Full |

## 2.2 K-map for L1

Ones at m7, m15; zeros at m0, m1, m3; all other cells ×.

| S3S2 \ S1S0 | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | 0 | 0 | 0 | × |
| 01 | × | × | 1 | × |
| 11 | × | × | 1 | × |
| 10 | × | × | × | × |

Grouping: one 8-cell group covering the two middle rows (all cells with S2 = 1). The group contains both 1s and only don't-cares otherwise, and no 0.

Result:

L1 = S2

## 2.3 K-map for L0

Ones at m3, m15; zeros at m0, m1, m7; all other cells ×.

| S3S2 \ S1S0 | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | 0 | 0 | 1 | × |
| 01 | × | × | 0 | × |
| 11 | × | × | 1 | × |
| 10 | × | × | × | × |

Groupings:

- 4-cell group: rows 00 and 10, columns 11 and 10 → all cells with

S2 = 0 AND S1 = 1 → term S1·S̄2 (covers m3; avoids the 0 at m7).

- 8-cell group: bottom two rows (S3 = 1) → term S3 (covers m15; m7 has

S3 = 0 so the 0 is not touched).

Result:

L0 = S1·S̄2 + S3

## 2.4 Gate implementation and S0

The encoder is built exactly from these minimized expressions using OR/AND/NOT gates, as the brief requires:

- L1 = S2 — direct connection (the K-map reduces it to a single literal).

- L0 = (S1 AND (NOT S2)) OR S3 — one NOT, one AND, one OR gate.

The K-maps eliminate S0 entirely: whenever S0 alone is wet the level is still "Empty" (00), and in every higher valid state S0 is implied by the thermometer property. S0 is therefore wired to an indicator LED (S0_WATER) in the circuit so the input is still observable, and this redundancy is stated explicitly rather than hidden.

Invalid-code behaviour (design decision): because invalid codes were used as don't-cares, a faulty sensor pattern maps deterministically through L1 = S2, L0 = S1·S̄2 + S3. For example 0100 (S2 stuck alone) reads as level 10 (50 %), and 1000 (S3 stuck alone) reads as level 01 (25 %). The system never crashes or produces an undefined display for any of the 16 input codes — Section 9 verifies all 16 exhaustively in simulation.

# 3. Step 2 — Comparator (Full Detection)

The comparator detects LEVEL = 11 by comparing the level code against the constant 11 — for a 2-bit equality against a constant of all-ones this reduces to a single AND gate:

| L1 | L0 | FULL |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

FULL = L1 · L0

Implemented with one 2-input AND gate. FULL feeds the overflow counter (Step 5) and is also brought out on the FULL output pin.

# 4. Step 3 — Pump Control Logic

Requirement: pump ON while the tank is below full, pump OFF at full.

PUMP = NOT(FULL) = NOT(L1 · L0) = NAND(L1, L0)

Implemented as a single 2-input NAND gate (the brief allows NAND or AND + NOT; NAND uses one gate instead of two).

| L1 | L0 | Level | PUMP |
| --- | --- | --- | --- |
| 0 | 0 | Empty | 1 (ON) |
| 0 | 1 | 25 % | 1 (ON) |
| 1 | 0 | 50 % | 1 (ON) |
| 1 | 1 | Full | 0 (OFF) |

The PUMP_ON LED in the circuit lights while the pump runs. Because PUMP is purely combinational on L1 L0, the pump reacts immediately when the FULL condition appears or clears — no clock edge is needed to stop filling.

# 5. Step 4 — Register (Level Memory)

Two positive-edge-triggered D flip-flops form a 2-bit register that stores the last valid tank level:

- FF_R1: D input = L1, output = R1

- FF_R0: D input = L0, output = R0

- Both clock inputs are driven by CLOCK; on each rising edge the register captures the current level code (R1 R0 ← L1 L0).

- Both asynchronous clear inputs are driven by RESET; asserting RESET forces R1 R0 = 00 immediately, independent of the clock.

Characteristic behaviour of each D flip-flop: Q(next) = D on the rising clock edge; Q holds its value between edges. This is what makes the display stable: the 7-segment digit only changes on a clock edge, even if the sensor inputs bounce between edges.

*Lab concepts applied: flip-flops, registers, clocking principles.*

# 6. Step 5 — Overflow Counter and Alarm

Requirement: trigger the alarm if FULL remains high for 3 clock cycles; the counter must reset when FULL = 0.

## 6.1 State design

A 2-bit synchronous counter (Q1 Q0) built from two D flip-flops counts rising clock edges only while FULL = 1. It counts 00 → 01 → 10 → 11 and then saturates at 11 (it holds 3 rather than wrapping to 0, so the alarm stays on while the overflow risk persists). When FULL = 0 the next-state logic forces 00 on the next edge, and RESET clears the counter asynchronously.

State table (Q1+ Q0+ = next state on a rising clock edge):

| FULL | Q1 | Q0 | Q1+ | Q0+ | Comment |
| --- | --- | --- | --- | --- | --- |
| 0 | × | × | 0 | 0 | counter cleared |
| 1 | 0 | 0 | 0 | 1 | first full cycle |
| 1 | 0 | 1 | 1 | 0 | second full cycle |
| 1 | 1 | 0 | 1 | 1 | third → alarm |
| 1 | 1 | 1 | 1 | 1 | saturate, alarm held |

## 6.2 Next-state derivation (D inputs)

For D flip-flops, D1 = Q1+ and D0 = Q0+. K-maps over (FULL, Q1, Q0):

D1 — ones at (1,0,1), (1,1,0), (1,1,1):

| FULL \ Q1Q0 | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | 1 | 1 |

Grouping: pair {FULL·Q0 columns 01,11} + pair {FULL·Q1 columns 11,10}:

D1 = FULL·(Q1 + Q0)

D0 — ones at (1,0,0), (1,1,0), (1,1,1):

| FULL \ Q1Q0 | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 0 | 1 | 1 |

Grouping: pair {FULL·Q̄0 columns 00,10} + pair {FULL·Q1 columns 11,10}:

D0 = FULL·(Q1 + Q̄0)

Q̄0 is taken directly from the flip-flop's complementary output, so no extra inverter is needed. Each D input costs one OR and one AND gate. Both flip-flops share CLOCK — the counter is fully synchronous.

## 6.3 Alarm

ALARM = Q1 · Q0

One AND gate. The counter reaches 11 on the third consecutive rising clock edge with FULL = 1, exactly as required, and the ALARM_LED lights. Both counter flip-flops are also cleared asynchronously by RESET.

*Lab concepts applied: counters, sequential logic.*

# 7. Step 6 — 7-Segment Display Decoder

The registered level R1 R0 (a 2-bit BCD value 0–3) drives a gate-built BCD-to-7-segment decoder. Segment naming is the standard a (top) … g (middle), active-high:

| R1 | R0 | Digit | a | b | c | d | e | f | g |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 2 | 1 | 1 | 0 | 1 | 1 | 0 | 1 |
| 1 | 1 | 3 | 1 | 1 | 1 | 1 | 0 | 0 | 1 |

With only two variables each K-map is a 2×2 grid; reading the groups gives:

| Segment | K-map cells that are 1 | Minimized expression | Gates used |
| --- | --- | --- | --- |
| a | 00, 10, 11 | a = R1 + R̄0 | OR (+ shared NOT) |
| b | all | b = 1 | constant 1 |
| c | 00, 01, 11 | c = R̄1 + R0 | OR (+ shared NOT) |
| d | 00, 10, 11 | d = a = R1 + R̄0 | reuse of segment a |
| e | 00, 10 | e = R̄0 | shared NOT |
| f | 00 | f = R̄1·R̄0 = NOR(R1, R0) | NOR |
| g | 10, 11 | g = R1 | wire |

Only two inverters (R̄1, R̄0) are needed and they are shared across segments; d reuses the segment-a signal because their columns are identical. The seven signals drive Logisim's 7-segment display component and are also brought out on pins SEG_A … SEG_G so the decoder can be verified in the truth table.

*Lab concepts applied: encoders/decoders, display interfacing.*

# 8. Boolean Derivations — Summary

Step-by-step, from requirement to final gate-level form:

L1 (level MSB)

- Requirement: L1 = 1 for levels 10 and 50 %/Full rows → minterms m7, m15.

- K-map with 11 don't-cares (Section 2.2) → single 8-cell group S2 = 1.

- L1 = S2.

L0 (level LSB)

- Requirement: L0 = 1 for 25 % and Full rows → minterms m3, m15.

- K-map (Section 2.3) → groups S1·S̄2 and S3.

- L0 = S1·S̄2 + S3 — implemented as (S1 AND NOT S2) OR S3.

FULL

- Equality comparison LEVEL = 11₂ → both bits must be 1.

- FULL = L1·L0 — one AND gate.

PUMP

- Requirement: ON below full, OFF at full → PUMP = F̄ULL.

- PUMP = NOT(L1·L0) — by De Morgan equivalent to L̄1 + L̄0.

- PUMP = NAND(L1, L0) — one gate.

ALARM

- Count consecutive FULL cycles with a 2-bit synchronous counter.

- Next-state K-maps (Section 6.2): D1 = FULL·(Q1+Q0),

D0 = FULL·(Q1+Q̄0).

- Alarm when count = 11₂ → ALARM = Q1·Q0.

Every expression above is implemented literally, gate for gate, in water_tank.circ, and the simulation results in Section 9 confirm each one.

# 9. Testing Scenarios and Simulation Results

Method. The delivered water_tank.circ was simulated headlessly with the real Logisim 2.7.1 engine using the included test harness (verification/TableDriver.java), which drives the input pins with scripted vectors and records every output pin after propagation. The observed values were compared bit-for-bit against an independent reference model (verification/verify.py). Result: 71 test steps, 0 mismatches. The full machine-generated table is in verification/verification_log.md; the scenarios below are extracts from that run, so every "Observed" entry is a real simulation result, not a prediction.

## 9.1 Exhaustive encoder / comparator / pump sweep

All 16 sensor codes were applied and clocked. Valid rows shown first:

| S3 S2 S1 S0 | Kind | L1 L0 | FULL | PUMP | Digit shown | Expected | Observed |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 0 0 0 | valid | 0 0 | 0 | 1 | 0 | same | ✔ match |
| 0 0 0 1 | valid | 0 0 | 0 | 1 | 0 | same | ✔ match |
| 0 0 1 1 | valid | 0 1 | 0 | 1 | 1 | same | ✔ match |
| 0 1 1 1 | valid | 1 0 | 0 | 1 | 2 | same | ✔ match |
| 1 1 1 1 | valid | 1 1 | 1 | 0 | 3 | same | ✔ match |
| 0 0 1 0 | invalid | 0 1 | 0 | 1 | 1 | same | ✔ match |
| 0 1 0 0 | invalid | 1 0 | 0 | 1 | 2 | same | ✔ match |
| 0 1 0 1 | invalid | 1 0 | 0 | 1 | 2 | same | ✔ match |
| 0 1 1 0 | invalid | 1 0 | 0 | 1 | 2 | same | ✔ match |
| 1 0 0 0 | invalid | 0 1 | 0 | 1 | 1 | same | ✔ match |
| 1 0 0 1 | invalid | 0 1 | 0 | 1 | 1 | same | ✔ match |
| 1 0 1 0 | invalid | 0 1 | 0 | 1 | 1 | same | ✔ match |
| 1 0 1 1 | invalid | 0 1 | 0 | 1 | 1 | same | ✔ match |
| 1 1 0 0 | invalid | 1 1 | 1 | 0 | 3 | same | ✔ match |
| 1 1 0 1 | invalid | 1 1 | 1 | 0 | 3 | same | ✔ match |
| 1 1 1 0 | invalid | 1 1 | 1 | 0 | 3 | same | ✔ match |

Every invalid (non-thermometer) code resolves to a defined level with no undefined outputs, as designed in Section 2.4.

## 9.2 Operational scenario (sequential behaviour)

Each row is one simulation step; "edge" marks a rising CLOCK edge.

| # | Scenario | Inputs (S3S2S1S0, RESET, CLOCK) | Expected | Observed |
| --- | --- | --- | --- | --- |
| B1 | Power-on reset | 0000, R=1, clock pulsed | Register 00, counter 00, digit 0, alarm off | ✔ match |
| B2 | Empty tank, clock edge | 0000, R=0, edge | Stores 00, digit 0, PUMP ON | ✔ match |
| B3 | Water at S0 only | 0001, edge | Still level 00, digit 0 | ✔ match |
| B4 | 25 % reached | 0011, edge | Stores 01, digit 1, pump ON | ✔ match |
| B5 | 50 % reached | 0111, edge | Stores 10, digit 2, pump ON | ✔ match |
| B6 | Tank FULL (pump ON→OFF) | 1111, edge 1 | FULL=1, PUMP OFF instantly, digit 3, counter 01 | ✔ match |
| B7 | FULL held, 2nd cycle | 1111, edge 2 | Counter 10, alarm still off | ✔ match |
| B8 | FULL held, 3rd cycle | 1111, edge 3 | Counter 11 → ALARM ON | ✔ match |
| B9 | FULL held, 4th cycle | 1111, edge 4 | Counter saturates at 11, alarm stays ON | ✔ match |
| B10 | Drained to 50 % (pump OFF→ON) | 0111, edge | FULL=0, PUMP ON instantly, counter clears 00, alarm OFF, digit 2 | ✔ match |
| B11 | Drained to 25 % | 0011, edge | Stores 01, digit 1 | ✔ match |
| B12 | Refill to FULL, 2 cycles | 1111, 2 edges | Counter reaches 10, alarm off | ✔ match |
| B13 | RESET while FULL | 1111, R=1, no edge | Counter clears to 00 immediately (async), alarm off; after release, next edge restarts count at 01 | ✔ match |
| B14 | Invalid code S2 alone | 0100, edge | Reads level 10, digit 2, no undefined outputs | ✔ match |
| B15 | Invalid code S3 alone | 1000, edge | Reads level 01, digit 1, no undefined outputs | ✔ match |
| B16 | Final reset | 0000, R=1 | Everything back to level 00, digit 0 | ✔ match |

## 9.3 Alarm timing detail (from the verified log)

| Clock edges with FULL=1 | Counter Q1 Q0 | ALARM |
| --- | --- | --- |
| 0 (just reached full) | 0 0 | 0 |
| 1 | 0 1 | 0 |
| 2 | 1 0 | 0 |
| 3 | 1 1 | 1 |
| 4, 5, … (held) | 1 1 | 1 |
| FULL drops, next edge | 0 0 | 0 |

The alarm fires on exactly the third consecutive clock cycle of FULL, holds while the risk persists, and clears when FULL drops or RESET is pressed.

# 10. System Explanation and Lab-Concept Mapping

How the system works, end to end. The four sensors produce a thermometer code that the Step-1 encoder compresses into the 2-bit level L1 L0. This level fans out three ways, exactly as in the required block diagram. First, the pump NAND gate keeps the pump running until the level reads 11, and stops it immediately at full — a purely combinational safety path that does not wait for a clock. Second, the comparator AND gate raises FULL when the level equals 11; FULL gates the 2-bit synchronous counter, which advances once per rising clock edge while the overflow risk lasts, and the alarm AND gate fires when the count reaches 3, i.e. FULL has persisted for three whole clock cycles. Third, the register's two D flip-flops snapshot the level on each clock edge, giving a stable "last valid level" that the gate-built decoder turns into a digit 0–3 on the 7-segment display. RESET asynchronously clears both the register and the counter, returning the system to a known state.

Lab concepts and where each is applied:

| Lab concept | Where it is applied in this project |
| --- | --- |
| Logic gates | NOT/AND/OR encoder, AND comparator, NAND pump, AND-OR counter logic, AND alarm, OR/NOR decoder |
| Boolean simplification | L1 = S2 and L0 = S1·S̄2 + S3 reduced from 4-variable tables; PUMP folded to one NAND by De Morgan |
| K-maps | Encoder (Sections 2.2–2.3), counter next-state (Section 6.2), decoder (Section 7) |
| Encoders / decoders | Step 1 is a priority-style level encoder; Step 6 is a BCD-to-7-segment decoder |
| 7-segment display | Driven digit 0–3 with active-high segments a–g (Section 7) |
| Flip-flops | Four positive-edge D flip-flops (register + counter) |
| Registers | 2-bit level register with shared clock and async clear (Step 4) |
| Counters | 2-bit synchronous saturating counter gated by FULL (Step 5) |
| Comparator | Equality detection LEVEL = 11 via FULL = L1·L0 (Step 2) |

# 11. How to Open and Run the Circuit

The circuit is delivered in Logisim `.circ` format (the brief allows Logisim or CircuitVerse; Logisim was chosen because its file format opens in both classic Logisim 2.7.1 and Logisim Evolution).

Open it:

- Install Logisim Evolution (recommended, free —

logisim-evolution on GitHub) or classic Logisim 2.7.1 (logisim-generic-2.7.1.jar from SourceForge, run with java -jar logisim-generic-2.7.1.jar).

- File → Open → circuit/water_tank.circ.

Drive it manually:

- Choose the Poke tool (the pointing-hand icon).

- Click the input pins S0…S3 to set a thermometer code (e.g. for 50 % set

S0 = S1 = S2 = 1), and click CLOCK twice (0→1→0) to give one clock cycle.

- Watch: the 7-segment digit, PUMP_ON LED, and after three clock cycles at

full, the ALARM_LED. Poke RESET to clear everything.

Reproduce the automated verification (optional):

cd verification

# put logisim-generic-2.7.1.jar in this folder as logisim.jar, then:

javac -cp logisim.jar TableDriver.java

python3 verify.py ../circuit/water_tank.circ

This regenerates verification_log.md (71 steps, expected 0 mismatches).

## Submission checklist (mapped to "What You Must Submit")

| Required item | Where it is in this submission |
| --- | --- |
| Logisim/CircuitVerse circuit file | circuit/water_tank.circ |
| Boolean derivations | Section 8 (details in Sections 2–7) |
| Truth tables | Sections 2.1, 3, 4, 6.1, 7 |
| K-map simplifications | Sections 2.2, 2.3, 6.2, 7 |
| Testing scenarios | Section 9 + verification/verification_log.md |
| System explanation | Sections 1 and 10 |